# Objective

This example demonstrates the use of PSoC 4 BLE Component to design a simple Find Me application.

# Overview

This code example uses the BLE component to design a simple Find Me application using the standard services defined by the Bluetooth SIG. In this code example, Find Me Profile instantiates an Immediate Alert Service which can be used to control the alert level of an indicator (buzzer, LED, etc.).

# Requirements

Tool: PSoC Creator 3.1, CySmart 1.0

Programming Language: C (GCC 4.8.4)

Associated Parts: All PSoC 4 BLE parts

Related Hardware: CY8CKIT-042-BLE Bluetooth® Low Energy (BLE) Pioneer Kit

# PSoC Creator Schematic

Figure 1. PSoC Creator Schematic

# Hardware Setup

BLE Pioneer Kit has the necessary hardware connections required for this lab. If you are using your own hardware, then connect the LED to P3.7 of PSoC 4 BLE.

# Operation

Figure 2. Firmware Flow

- main() function: This is the central function which performs the initialization of the BLE stack, PWM for the LED control and executes the necessary routines to process the BLE events and maintain the connection.

In the initial section of the main() function, the API CyBle_Start(StackEventHandler) is called to start the BLE Component and register a callback to the stack event handler. Note that the callback function can assume any name – in this project, we used StackEventHandler. Once the system is initialized, main() function continuously operates in a while(1) loop executing CyBle_ProcessEvents(). CyBle_ProcessEvents processes the events received by the BLE stack and enables application layer to use them and take the appropriate action

- StackEventHandler() function: This function handles the common events generated for the BLE Stack. For example, the event CYBLE_EVT_STACK_ON is received when the Stack is initialized and turned ON and the event CYBLE_EVT_GAP_DEVICE_DISCONNECTED is received when the BLE connection gets disconnected.

- IasEventHandler() function: This function handles the events for Immediate Alert Service. As a part of the event, it receives the alert levels which are used to drive the LED as per Table 1.

Table 1: Alert Level vs LED Blink Rate

| Alert Level | PWM Duty Cycle | LED Status |
| --- | --- | --- |
| NO_ALERT | 100% | Always OFF |
| MILD_ALERT | 50% | LED toggling every second |
| HIGH_ALERT | 0% | Always ON |

Note: LED pin is connected to the inverted terminal (line_n) of PWM, thus 100% duty cycle of PWM corresponds to LED always OFF.

## Build and Program the device

This section shows how to build the project and program the PSoC 4 BLE device. If you are using a development kit with a built-in programmer (BLE Pioneer Kit, for example), connect the BLE Pioneer Baseboard to your computer using the USB Standard-A to Mini-B cable. For other kits, refer to the kit user guide.

If you are developing on your own hardware, you need a hardware debugger, for example, a Cypress CY8CKIT-002 MiniProg3.

- On PSoC Creator, select Build > Clean and Build PSoC 4BLE Find Me, as shown in Figure 23.

Figure 23. Build Project

- On successful build, total flash and SRAM usage is reported, as shown in Figure 24.

Figure 24. Build Succeeded

- Select Debug > Select Debug Target, as shown in Figure 25.

Figure 25. Selecting Debug Target

In the Select Debug Target dialog box, click Port Acquire, and then click Connect, as shown in Figure 26. Click OK to close the dialog box.

Figure 26. Connecting to a Device

If you are using your own hardware, make sure the Port Setting configuration under Select Debug Target window for your programming hardware is configured as per your setup.

Select Debug > Program to program the device with the project, as shown in Figure 27.

Figure 27. Programming the Device

You can view the programming status on the PSoC Creator status bar (lower-left corner of the window), as shown in Figure 28.

Figure 28. Programming Status

# Testing

- Plug the BLE-USB Bridge (included with the BLE Pioneer Kit) in your computer’s USB port.

- On your computer, launch CySmart 1.0. It is located in the All Programs -> Cypress -> CySmart folder in the Windows start menu. The tool opens up and asks you to Select BLE Dongle Target. Select the Cypress BLE Dongle (COMxx) and click Connect, as shown in Figure 3.

Figure 3: CySmart 1.0: Select BLE Dongle Target

- When the BLE-USB Bridge is connected, click on Start Scan to find your BLE device. See Figure 4.

Figure 4: Finding a BLE Device

- The scanning stops automatically once all the nearby devices are known. The tool lists all the nearby devices in the Discovered devices section.

- Click on your device name to see the Advertisement data and Scan response data packets on the right. See Figure 5.

Figure 5: Checking Discovery Details of a Connected BLE Device

- Click Connect as seen in Figure 5 to connect to the device.

- The tool will now open a separate tab for the device. Click Discover All Attributes to list all the Attributes in the device, with their respective UUIDs and descriptions. See Figure 6.

Figure 6: Discovering Attributes of a Connected BLE Device

- Click on any row in the list of Attributes to see its details on the right. To read an Attribute’s value, click Read Value on the right as shown in Figure 7.

Figure 7: Reading Attribute Value

- Locate the Alert Level Attribute for the Immediate Alert Service. On the right, write a value of 1 to start blinking the LED.  See Figure 8.

Figure 8: Writing Attribute Value

- Write a value of 2 to keep the LED always on.

- Write a value of 0 to turn off the LED.

# Related Documents

Table 2 lists all relevant application notes, code examples, knowledge base articles, device datasheets, and Component / user module datasheets.

Table 2. Related Documents

| Document | Title | Comment |
| --- | --- | --- |
| AN91267 | Getting Started with PSoC 4 BLE | Provides an introduction to PSoC 4 BLE device that integrates a Bluetooth Low Energy radio system along with programmable analog and digital resources. |
| AN91445 | Antenna Design Guide | Provides guidelines on how to design an antenna for BLE applications. |